Marbelite Co., Inc.

Volume

1

ELTEC NTCE-17 Timeclock E-Z Setup Manual

E-Z Setup Manual

MArbelite Co., Inc.

ELTEC NTCE-17 E-Z Setup

 Marbelite Co., Inc.

1913 Atlantic Ave.

Manasquan, NJ 08736

Chapter

1

Terminology and Basic Theory Of Operation

Terminology

the  basic terminology that you should be familiar with in this manual are the following three words.

- “Step”   A Step is a time to tell the timeclock  to turn on or off.

- “Exception” A Exception is a date or series of dates when you want the timeclock  to run a “Program” other than the normal operation.

- “Program” A Program is a series of steps that runs the timeclock through its on and off operations. (Typically a timeclock will have 3 programs consisting of several steps, these are usually Program 00, 01, and 02)

Basic Theory Of Operation

Programming a timeclock consists of about 4 basic operations they are as follows:

- Setting the time and date of the timeclock

- Figuring out what programs you will be running.  (Ex: regular session- 00, day off- 01, ½ day- 02 )

- Setting the steps for each program

- Setting your exceptions from the regular session program (00)

How To Program Your Timeclock

Chapter

2

In the following exercises you will learn how to

- Set the time

- Set the date

- Set several steps linked to a program

- Set several exceptions linked to a program

# Setting the time

The following list will explain to you how to set the time on the timeclock. All time settings are done in a 12 hour format, ex: 08:00. If you should make a mistake in setting the time press C  and start the procedure over again.

- Press A11

- Enter time in 12 hour format (ex: 08:00)

- Press A or am or D for pm

- Press B to set the time

# Setting the date

The following list will explain to you how to set the date on the timeclock. If you should make a mistake in setting the date, press C and start the procedure over again.

- Press A21

- Enter the date

- Press B to set the date

# Setting the Steps

The following list will explain to you how to set steps in the timeclock. If you make a mistake in the procedure ,press C, and then press A85  followed by D to clear the steps.

- Press A86

- Enter your program number (In this exercise we will use the default 00. Marbelite recommends using 00 for your regular session, and 02 for ½ day session.)

- Press B

- Enter the time you wish the timeclock to turn on in 12 hour format (ex: 08:00)

- Press A for am or D for pm

- Press B to set the time

- Enter 1 (This will tell the relay to turn on at this time. Your screen should look as such “relays on=1”)

- Press B

- Press 9 (This will set the timeclock to run this step only on weekdays.)

- Press B to enter this step completely

Now you will enter the time to turn the timeclock off. At this point the timeclock should be in the first screen of step 2.

- Enter your program number (Again this will be the default 00)

- Press B

- Set the time you wish the timeclock to turn off

- Press A for am or D for pm

- Press B to set the time

- Leave this screen blank (This will tell the timeclock to turn off. Your screen should look as such: “relays on=”)

- Press 9 (This will set the timeclock to run this step only on weekdays.)

- Press B

From here you would continue to set the steps in the above manner, turning the timeclock on and off. The only change you would make would be the time or the program number. (Again 00 for regular session, 02 for ½ day.)

# Setting the Exceptions

The following list will explain to you how to set exceptions in the timeclock. If you make a mistake in the procedure ,press C, and then press A75  followed by D to clear the exceptions.

- Press A76

- Enter the program number (In this exercise we will use 01 as this will keep the timeclock from coming on)

- Press B

- Enter your start date (ex: 11/30/01)

- Press B

- Enter your end date (ex: 12/7/01)

- Press B

The timeclock should now be showing exception 2 and asking you for a program number. This time we will enter 02 for a ½ day.

- Press B

- Enter your start date (ex: 02/18/02)

- Press B

- Enter your end date (ex: 02/18/02)

From here you would continue to set the exceptions in the above manner. When you are finished press C to return back to showing the time.

How To Clear Exceptions and Steps and Add New Ones

Chapter

3

In the following exercises you will learn how to

- Clear Exceptions

- Clear Steps

# Clearing All Exceptions

The following list will explain to you how to clear all exceptions.

- Press A75

- Press D

- Continue from Chapter 2 “Settting the Exceptions”

# Clearing All Steps

The following list will explain to you how to clear all exceptions.

- Press A8

- Press D

- Continue from Chapter 2 “Setting the Steps

If you should requre any other assistance, please call us at 732.292.2100 or e-mail info@marbsignal.com